Release Notes – E370 V1.6 July 25, 2021

To install the software

a) copy the e370_fmw.bin file to the ROOT directory of your uSD card. The card MUST BE FAT32 formatted Use 32GB or smaller cards).

b) turn off power to the E370, insert the uSD card, and while HOLDING DOWN the encoder knob, apply power.

c) release the knob. The Bootloader should run and detect the firmware file. It will ask if you really want to do this, select YES and click. The new firmware will load.

d) remove the uSD card when the E370 reboots.

- NOTE: this update required changes to the EEPROM memory so your patches and wave table names will be erased. The wavetables will still be accessible but will revert to default names.

- Encoder Acceleration - turning the encoder quickly now causes "click acceleration" which helps move across long lists with less turning.

- Adjusted quantizer to yield A440 at 0 cents offset.

- HPF option for FM - a new item on the Frequency page provides four HPF corners. This works in both Expo and Linear FM modes and eliminates the frequency offset that's observed mainly in the high sensitivity linear FM modes. The settings are:

  - Off (default) – No high-pass action. Fully DC coupled.

  - 0.1Hz – High pass corner at 0.1Hz

  - 1Hz – High pass corner at 1Hz

  - 10Hz – High pass corner at 10Hz

- Added OFF option to FM source setting to improve chord mode tracking. This completely disables any FM input to ensure exact tracking of chords vs the root source.

- Patches - Patch page now supports name editing with a simple navigation process. Context-sensitive help text is provided to explain what's going on.

- New Morph+Chord mode - similar to E352’s Morph+Chord, but fewer parameters and options. Defaults to Z morph and Chord on X,Y params and Spread is defaulted to full-on. Spread can be manually set or routed to another input if needed.

- New Morph+Sub mode - similar to E352 Morph+Blend. Provides blending between two waveforms plus sub/super oscillator mode with octave offsets. Note - 2nd waveform uses same bank / Z-morph as first, but can be other types of wave (sine/saw/etc).

- Added per-voice unipolar output option (0-5V swing) to mixer page – helps when using the E370 for LFOs. Basically cuts the amplitude in half and offsets it so that 0V is the minimum level.

- New Moses page (under About page) – all your cat-related content.

- New option on Freq Range setting for Zero frequency + Phase set to 0deg for more predictable outputs when scanning wavetables with a CV.

- New feature to allow copying voice settings between voices. Accessible from the top-level Settings page (bottom left “Copy” item). Note that to prevent accidentally overtaxing the CPU all cloud modes are limited to 4 oscillators max when copied.

- New setting for Morph + Wavefolder to allow hard clipping instead of wave folding.

- New logic for estimating CPU loading and preventing GUI from locking up. Note that this may sometimes force cloud modes from 8 down to 4 oscillators if overload is detected.

- Add A440 option to 1V/Oct source setting. This bypasses all pitch controls (1V/Oct, chord, FM) and forces the voice to a pitch of A440. Morphing / clouds / etc still operate, so the actual perceived pitch may deviate.

- Patch Number:Name display now persists across a power cycle. Previously, powe cycling would clear any displayed patch info even if the settings had not been altered.

Four chord tables are available in ROM for use with the new Morph+Chord mode.

- "3-note" contains 64 chords of 3 notes spanning an octave.

- "4-note" contains 60 chords of 4 notes spanning about 3/4 octave

- "Harmonic" contains 64 chords of 4 notes using a Just Intonation scheme.

- "Stradella" contains 64 chords of 3 notes based on the Stradella accordion button layout.

The following are complete lists of the built-in chord tables. They consist of four columns of numbers describing the interval offset from the root note where a value of 1.0F is one equal temperament semitone.

3-note chords:

| Row | Osc 1 | Osc 2 | Osc 3 |
| --- | --- | --- | --- |
| 1 | 0.00 | 2.00 | 4.00 |
| 2 | 0.00 | 2.00 | 5.00 |
| 3 | 0.00 | 2.00 | 6.00 |
| 4 | 0.00 | 2.00 | 7.00 |
| 5 | 0.00 | 2.00 | 8.00 |
| 6 | 0.00 | 2.00 | 9.00 |
| 7 | 0.00 | 2.00 | 10.00 |
| 8 | 0.00 | 2.00 | 11.00 |
| 9 | 0.00 | 2.00 | 12.00 |
| 10 | 0.00 | 2.00 | 13.00 |
| 11 | 0.00 | 2.00 | 14.00 |
| 12 | 0.00 | 3.00 | 5.00 |
| 13 | 0.00 | 3.00 | 6.00 |
| 14 | 0.00 | 3.00 | 7.00 |
| 15 | 0.00 | 3.00 | 8.00 |
| 16 | 0.00 | 3.00 | 9.00 |
| 17 | 0.00 | 3.00 | 10.00 |
| 18 | 0.00 | 3.00 | 11.00 |
| 19 | 0.00 | 3.00 | 12.00 |
| 20 | 0.00 | 3.00 | 13.00 |
| 21 | 0.00 | 3.00 | 14.00 |
| 22 | 0.00 | 4.00 | 6.00 |
| 23 | 0.00 | 4.00 | 7.00 |
| 24 | 0.00 | 4.00 | 8.00 |
| 25 | 0.00 | 4.00 | 9.00 |
| 26 | 0.00 | 4.00 | 10.00 |
| 27 | 0.00 | 4.00 | 11.00 |
| 28 | 0.00 | 4.00 | 12.00 |
| 29 | 0.00 | 4.00 | 13.00 |
| 30 | 0.00 | 4.00 | 14.00 |
| 31 | 0.00 | 5.00 | 7.00 |
| 32 | 0.00 | 5.00 | 8.00 |
| 33 | 0.00 | 5.00 | 9.00 |
| 34 | 0.00 | 5.00 | 10.00 |
| 35 | 0.00 | 5.00 | 11.00 |
| 36 | 0.00 | 5.00 | 12.00 |
| 37 | 0.00 | 5.00 | 13.00 |
| 38 | 0.00 | 5.00 | 14.00 |
| 39 | 0.00 | 6.00 | 8.00 |
| 40 | 0.00 | 6.00 | 9.00 |
| 41 | 0.00 | 6.00 | 10.00 |
| 42 | 0.00 | 6.00 | 11.00 |
| 43 | 0.00 | 6.00 | 12.00 |
| 44 | 0.00 | 6.00 | 13.00 |
| 45 | 0.00 | 6.00 | 14.00 |
| 46 | 0.00 | 7.00 | 9.00 |
| 47 | 0.00 | 7.00 | 10.00 |
| 48 | 0.00 | 7.00 | 11.00 |
| 49 | 0.00 | 7.00 | 12.00 |
| 50 | 0.00 | 7.00 | 13.00 |
| 51 | 0.00 | 7.00 | 14.00 |
| 52 | 0.00 | 8.00 | 10.00 |
| 53 | 0.00 | 8.00 | 11.00 |
| 54 | 0.00 | 8.00 | 12.00 |
| 55 | 0.00 | 8.00 | 13.00 |
| 56 | 0.00 | 8.00 | 14.00 |
| 57 | 0.00 | 9.00 | 11.00 |
| 58 | 0.00 | 9.00 | 12.00 |
| 59 | 0.00 | 9.00 | 13.00 |
| 60 | 0.00 | 9.00 | 14.00 |
| 61 | 0.00 | 10.00 | 12.00 |
| 62 | 0.00 | 10.00 | 13.00 |
| 63 | 0.00 | 10.00 | 14.00 |
| 64 | 0.00 | 11.00 | 13.00 |

4-note chords

| Row | Osc 1 | Osc 2 | Osc 3 | Osc 4 |
| --- | --- | --- | --- | --- |
| 1 | 0.00 | 2.00 | 4.00 | 6.00 |
| 2 | 0.00 | 2.00 | 4.00 | 7.00 |
| 3 | 0.00 | 2.00 | 4.00 | 8.00 |
| 4 | 0.00 | 2.00 | 4.00 | 9.00 |
| 5 | 0.00 | 2.00 | 4.00 | 10.00 |
| 6 | 0.00 | 2.00 | 4.00 | 11.00 |
| 7 | 0.00 | 2.00 | 5.00 | 7.00 |
| 8 | 0.00 | 2.00 | 5.00 | 8.00 |
| 9 | 0.00 | 2.00 | 5.00 | 9.00 |
| 10 | 0.00 | 2.00 | 5.00 | 10.00 |
| 11 | 0.00 | 2.00 | 5.00 | 11.00 |
| 12 | 0.00 | 2.00 | 6.00 | 8.00 |
| 13 | 0.00 | 2.00 | 6.00 | 9.00 |
| 14 | 0.00 | 2.00 | 6.00 | 10.00 |
| 15 | 0.00 | 2.00 | 6.00 | 11.00 |
| 16 | 0.00 | 2.00 | 7.00 | 9.00 |
| 17 | 0.00 | 2.00 | 7.00 | 10.00 |
| 18 | 0.00 | 2.00 | 7.00 | 11.00 |
| 19 | 0.00 | 2.00 | 8.00 | 10.00 |
| 20 | 0.00 | 2.00 | 8.00 | 11.00 |
| 21 | 0.00 | 2.00 | 9.00 | 10.00 |
| 22 | 0.00 | 2.00 | 9.00 | 11.00 |
| 23 | 0.00 | 3.00 | 5.00 | 7.00 |
| 24 | 0.00 | 3.00 | 5.00 | 8.00 |
| 25 | 0.00 | 3.00 | 5.00 | 9.00 |
| 26 | 0.00 | 3.00 | 5.00 | 10.00 |
| 27 | 0.00 | 3.00 | 5.00 | 11.00 |
| 28 | 0.00 | 3.00 | 6.00 | 8.00 |
| 29 | 0.00 | 3.00 | 6.00 | 9.00 |
| 30 | 0.00 | 3.00 | 6.00 | 10.00 |
| 31 | 0.00 | 3.00 | 6.00 | 11.00 |
| 32 | 0.00 | 3.00 | 7.00 | 9.00 |
| 33 | 0.00 | 3.00 | 7.00 | 10.00 |
| 34 | 0.00 | 3.00 | 7.00 | 11.00 |
| 35 | 0.00 | 3.00 | 8.00 | 10.00 |
| 36 | 0.00 | 3.00 | 8.00 | 11.00 |
| 37 | 0.00 | 3.00 | 9.00 | 10.00 |
| 38 | 0.00 | 3.00 | 9.00 | 11.00 |
| 39 | 0.00 | 4.00 | 6.00 | 8.00 |
| 40 | 0.00 | 4.00 | 6.00 | 9.00 |
| 41 | 0.00 | 4.00 | 6.00 | 10.00 |
| 42 | 0.00 | 4.00 | 6.00 | 11.00 |
| 43 | 0.00 | 4.00 | 7.00 | 9.00 |
| 44 | 0.00 | 4.00 | 7.00 | 10.00 |
| 45 | 0.00 | 4.00 | 7.00 | 11.00 |
| 46 | 0.00 | 4.00 | 8.00 | 10.00 |
| 47 | 0.00 | 4.00 | 8.00 | 11.00 |
| 48 | 0.00 | 4.00 | 9.00 | 10.00 |
| 49 | 0.00 | 4.00 | 9.00 | 11.00 |
| 50 | 0.00 | 5.00 | 7.00 | 9.00 |
| 51 | 0.00 | 5.00 | 7.00 | 10.00 |
| 52 | 0.00 | 5.00 | 7.00 | 11.00 |
| 53 | 0.00 | 5.00 | 8.00 | 10.00 |
| 54 | 0.00 | 5.00 | 8.00 | 11.00 |
| 55 | 0.00 | 5.00 | 9.00 | 10.00 |
| 56 | 0.00 | 5.00 | 9.00 | 11.00 |
| 57 | 0.00 | 6.00 | 8.00 | 10.00 |
| 58 | 0.00 | 6.00 | 8.00 | 11.00 |
| 59 | 0.00 | 6.00 | 9.00 | 11.00 |
| 60 | 0.00 | 7.00 | 9.00 | 11.00 |

Harmonic Chords

| Row | Osc 1 | Osc 2 | Osc 3 | Osc 4 |
| --- | --- | --- | --- | --- |
| 1 | 0.00 | 2.98 | 5.51 | 12.00 |
| 2 | 0.00 | 2.98 | 7.02 | 13.05 |
| 3 | 0.00 | 2.98 | 8.41 | 14.04 |
| 4 | 0.00 | 2.98 | 9.06 | 14.98 |
| 5 | 0.00 | 3.86 | 9.69 | 15.86 |
| 6 | 0.00 | 3.86 | 10.88 | 16.71 |
| 7 | 0.00 | 3.86 | 12.00 | 17.51 |
| 8 | 0.00 | 3.86 | 13.05 | 19.02 |
| 9 | 0.00 | 3.86 | 14.04 | 20.41 |
| 10 | 0.00 | 7.02 | 14.98 | 21.06 |
| 11 | 0.00 | 7.02 | 15.86 | 21.69 |
| 12 | 0.00 | 7.02 | 16.71 | 22.88 |
| 13 | 0.00 | 7.02 | 17.51 | 24.00 |
| 14 | 0.00 | 7.02 | 19.02 | 25.05 |
| 15 | 0.00 | 7.02 | 20.41 | 26.04 |
| 16 | 0.00 | 7.02 | 21.06 | 26.98 |
| 17 | 7.02 | 9.68 | 21.69 | 27.86 |
| 18 | 7.02 | 9.68 | 22.88 | 28.71 |
| 19 | 7.02 | 9.68 | 12.00 | 29.51 |
| 20 | 7.02 | 9.68 | 13.05 | 31.02 |
| 21 | 7.02 | 9.68 | 14.04 | 32.41 |
| 22 | 7.02 | 9.68 | 14.98 | 33.06 |
| 23 | 7.02 | 9.68 | 15.86 | 33.69 |
| 24 | 7.02 | 9.68 | 16.71 | 34.88 |
| 25 | 7.02 | 12.00 | 17.51 | 24.00 |
| 26 | 7.02 | 12.00 | 19.02 | 25.05 |
| 27 | 7.02 | 12.00 | 20.41 | 26.04 |
| 28 | 7.02 | 12.00 | 21.06 | 26.98 |
| 29 | 7.02 | 12.00 | 21.69 | 27.86 |
| 30 | 7.02 | 12.00 | 22.88 | 28.71 |
| 31 | 7.02 | 12.00 | 24.00 | 29.51 |
| 32 | 7.02 | 12.00 | 25.05 | 31.02 |
| 33 | 7.02 | 12.00 | 26.04 | 32.41 |
| 34 | 7.02 | 12.00 | 26.98 | 33.06 |
| 35 | 7.02 | 12.00 | 27.86 | 33.69 |
| 36 | 12.00 | 19.02 | 28.71 | 34.88 |
| 37 | 12.00 | 20.41 | 29.51 | 36.00 |
| 38 | 12.00 | 21.06 | 31.02 | 37.05 |
| 39 | 12.00 | 21.69 | 32.41 | 38.04 |
| 40 | 12.00 | 22.88 | 33.06 | 38.98 |
| 41 | 12.00 | 24.00 | 33.69 | 39.86 |
| 42 | 12.00 | 25.05 | 34.88 | 40.71 |
| 43 | 12.00 | 26.04 | 24.00 | 41.51 |
| 44 | 12.00 | 26.98 | 25.05 | 43.02 |
| 45 | 12.00 | 27.86 | 26.04 | 44.41 |
| 46 | 15.86 | 28.71 | 26.98 | 45.06 |
| 47 | 15.86 | 29.51 | 27.86 | 45.69 |
| 48 | 15.86 | 24.00 | 28.71 | 36.00 |
| 49 | 15.86 | 25.05 | 29.51 | 37.05 |
| 50 | 15.86 | 26.04 | 31.02 | 38.04 |
| 51 | 15.86 | 26.98 | 32.41 | 38.98 |
| 52 | 15.86 | 27.86 | 33.06 | 39.86 |
| 53 | 19.02 | 28.71 | 33.69 | 40.71 |
| 54 | 19.02 | 29.51 | 34.88 | 41.51 |
| 55 | 19.02 | 31.02 | 36.00 | 43.02 |
| 56 | 19.02 | 32.41 | 37.05 | 44.41 |
| 57 | 19.02 | 33.06 | 38.04 | 45.06 |
| 58 | 19.02 | 33.69 | 38.98 | 45.69 |
| 59 | 19.02 | 34.88 | 39.86 | 46.88 |
| 60 | 19.02 | 36.00 | 40.71 | 48.00 |
| 61 | 19.02 | 37.05 | 41.51 | 49.05 |
| 62 | 19.02 | 38.04 | 43.02 | 49.05 |
| 63 | 19.02 | 38.98 | 44.41 | 49.05 |
| 64 | 19.02 | 39.86 | 45.06 | 49.05 |

Stradella chords

| Row | Osc 1 | Osc 2 | Osc 3 |
| --- | --- | --- | --- |
| 1 | 0.00 | 4.00 | 7.00 |
| 2 | 0.00 | 3.00 | 7.00 |
| 3 | 0.00 | 4.00 | 10.00 |
| 4 | 0.00 | 3.00 | -3.00 |
| 5 | 7.00 | 11.00 | 14.00 |
| 6 | 7.00 | 10.00 | 14.00 |
| 7 | 7.00 | 11.00 | 17.00 |
| 8 | 7.00 | 10.00 | 4.00 |
| 9 | 2.00 | 6.00 | 9.00 |
| 10 | 2.00 | 5.00 | 9.00 |
| 11 | 2.00 | 6.00 | 12.00 |
| 12 | 2.00 | 5.00 | -1.00 |
| 13 | 9.00 | 13.00 | 16.00 |
| 14 | 9.00 | 12.00 | 16.00 |
| 15 | 9.00 | 13.00 | 19.00 |
| 16 | 9.00 | 12.00 | 6.00 |
| 17 | 4.00 | 8.00 | 11.00 |
| 18 | 4.00 | 7.00 | 11.00 |
| 19 | 4.00 | 8.00 | 14.00 |
| 20 | 4.00 | 7.00 | 1.00 |
| 21 | 11.00 | 15.00 | 18.00 |
| 22 | 11.00 | 14.00 | 18.00 |
| 23 | 11.00 | 15.00 | 21.00 |
| 24 | 11.00 | 14.00 | 8.00 |
| 25 | 6.00 | 10.00 | 13.00 |
| 26 | 6.00 | 9.00 | 13.00 |
| 27 | 6.00 | 10.00 | 16.00 |
| 28 | 6.00 | 9.00 | 3.00 |
| 29 | 1.00 | 5.00 | 8.00 |
| 30 | 1.00 | 4.00 | 8.00 |
| 31 | 1.00 | 5.00 | 11.00 |
| 32 | 1.00 | 4.00 | -2.00 |
| 33 | 8.00 | 12.00 | 15.00 |
| 34 | 8.00 | 11.00 | 15.00 |
| 35 | 8.00 | 12.00 | 18.00 |
| 36 | 8.00 | 11.00 | 5.00 |
| 37 | 3.00 | 7.00 | 10.00 |
| 38 | 3.00 | 6.00 | 10.00 |
| 39 | 3.00 | 7.00 | 13.00 |
| 40 | 0.00 | 6.00 | 3.00 |
| 41 | 10.00 | 14.00 | 17.00 |
| 42 | 10.00 | 13.00 | 17.00 |
| 43 | 10.00 | 14.00 | 20.00 |
| 44 | 10.00 | 13.00 | 7.00 |
| 45 | 5.00 | 9.00 | 12.00 |
| 46 | 5.00 | 8.00 | 12.00 |
| 47 | 5.00 | 9.00 | 15.00 |
| 48 | 5.00 | 8.00 | 2.00 |
| 49 | 12.00 | 16.00 | 19.00 |
| 50 | 12.00 | 15.00 | 19.00 |
| 51 | 12.00 | 16.00 | 22.00 |
| 52 | 12.00 | 15.00 | 9.00 |
| 53 | 7.00 | 11.00 | 14.00 |
| 54 | 7.00 | 10.00 | 14.00 |
| 55 | 7.00 | 11.00 | 17.00 |
| 56 | 7.00 | 10.00 | 4.00 |
| 57 | 2.00 | 6.00 | 9.00 |
| 58 | 2.00 | 5.00 | 9.00 |
| 59 | 2.00 | 6.00 | 12.00 |
| 60 | 2.00 | 5.00 | -1.00 |
| 61 | 9.00 | 13.00 | 16.00 |
| 62 | 9.00 | 12.00 | 16.00 |
| 63 | 9.00 | 13.00 | 19.00 |
| 64 | 9.00 | 12.00 | 6.00 |